Q.1)

ANS: Steps:

- Get 8 pc  , 4 switchs and 4 router

- Connect pc 1 and 2 to switch 1 (using straight cable)

- Connect pc 3 and 4 to switch 2(using straight cable)

- Connect pc 5 and 6 to switch 3(using straight cable)

- Connect pc 7 and 8 to switch 4(using straight cable)

- Connect router 1 to 2 (using cross cable)

- Connect router 2 to 3 (using cross cable)

- Connect router 3 to 4 (using cross cable)

- Assign ip to PCs keep it in class a ( 10.0.0.0 – 13.0.0.0 networks) and gateway

- In each router assign ip to the gigabitEnternet (make 1 network of 2 routers )

- Network for router ( 14.0.0.0 , 15.0.0.0 and 16.0.0.0) also assign ip to its interfaces according to network

- Network for switch (10.0.0.0 , 11.0.0.0 , 12.0.0.0 and 13.0.0.0)

- Now for each router go to static

  - First input = what network it is going to

  - Send input = subnetmask ( 255.0.0.0 for all)

  - 3rd input = through which port ( next router port ip assigen)

Q.2)

Steps:

- Get 1 switch

- Get 6 PCs

- Assign each pc ip according to its network

- Let pc 1-2 is HR

- Pc 3-4 is ADMin

- Pc 5-6 is Finance

- Connect:

  - fa0/1 & fa0/2 to HR PCs

  - fa0/3 & fa0/2 to ADMIN PCs

  - fa0/5 & fa0/6 to Finance PCs

- Now open switch and then CLI and run cmds given in the screenshots